welcome to this educational video on design failure modes and effects analysis I'll go through an example here so that we can understand how to fill out some of the columns so the important thing to know is that this is an analytical risk assessment tool that we use a design engineering it is a living

document if information becomes available, we need to update it and we need to revise it accordingly it seems really to prevent recalls warranty problems and any safety issues that would cause a negative reflection on the company's reputation in the marketplace and also have a negative effect on the business and sometimes businesses go out of business when the issues are too much for them to handle we want to avoid that this is what the DFMEA is trying to avoid a companies typically use diverse teams to complete

DFMEAS in my experience I've worked on DFMEAS with of course design is at the Table, manufacturing has been at the table, marketing, sales supply chain analysts are sometimes at the table we also want to ensure that the designs are safe robust and reliable we want to have a good quality product in the market and

we want to make sure that we have a streamlined product development process to get there if we don't have a good reliable product and get it on the market quickly we I can pretty much guarantee that the competition will do that and then it will cause us a problem with the market share and we want to anticipate any problems before we go into testing because the longer we wait to find an issue the more costly it is so let's take a look at the tool all right here's the DFMEA template that is currently in your DMADVR tool box, there are going to be some examples that I'll post on canvas they will look slightly different some of the columns but it's the process is the same I've just tried to updated as much as possible using those current template on available that industry might be using and accordance and I use this in accordance with AIAG the Automotive Industry Action Group so at the top of this we need to fill out the title block I can't stress how important that is because this is part of your design documentation and that information is critical in terms of filing this putting it in order organizing your design documentation so we can start with an example to explain each one of these columns so let's take a look oh wait we're going to look at this ballpoint pen so at the top says that's indeed what we're looking at the ballpoint pen the model the year over in the right of the title block we have the team that was it was prepared by and this is supposed to be page 1 of 1. I'm sorry that got cut off DFMEA date the DFMEA date 2/10/19 and then the Rev or the revision so this is revision to a Rev 2 so which means we already have had a Rev 1 so now we're updating so we're actually updating this document so the item we're going to be looking at is a pen tip so this is the part the sub function the sub-assembly or the component that we're looking at so pen tip is what we've decided so now we have to look at the function what's the primary function of this item that we've selected well we've two functions that we determined it disburses proper ink amounts onto the paper and the ball run smoothly so the next item that we need to think about is those potential failure modes remember those are the potential modes associated with this

function so the failure modes that can happen here and what way could the item fail to perform essentially its intended function well for the first function it's not enough ink or too much ink the second function is it's not smoothly the next column we have to take under of account is the potential failure effects so what are the impacts of this on the customer what will the customer see ,feel or experience are there any regulations that we have to think about or is there anything that we'll have an issue with other parts or subsystem surrounding our particular item so what is the impact to the customer so for these two failure modes we looked and said the pen skip or required heavy pressure while writing was the effects that would happen with not enough ink as our failure mode and then for too much ink is our failure mode we would see globs or drip left behind the letters so this is again what the customer is going to experience for not smoothly we would have inconsistent line skip or globs left behind the next thing we need to do is to rank or address the severity of this effect on our customer by giving it a numerical value we may remember these DFMEA

rankings or this DFMEA table is sometimes referred to and so here are the rankings remember 1 through 10. 1 at the bottom 10 at the top. the 1 is the least amount of severity and 10 is the worst hazardous without warning or personal safety and this ranking is available in Univ met in your DMADVR toolbox so let's take a look at how our team ranked our effects alright so the first two were ranked at a 7 so a 7 is reduced primary function or performance reduce primary function performance and then an 8 which is a little bit worse than is loss of primary function so maybe you don't agree with these rankings but these are the rankings that the team came up with again this is a team tool for you to use and discuss what you think you're brainstorming comes up with and then also how you feel the severity and these are things to think about how you might justify what your ranking is and in accordance with your product and your item that you're addressing let's

continue down this path so the next thing we have to look at the next column is the potential cause or the mechanism of the failure mode so we're going back to this failure mode and we're looking at what is the cause of that failure mode how can the failure occur and what could cause that item to fail in this

particular manner so we need to identify all the potential causes of the failure mode and again this is just a brainstorming activity with your team, and we'll just look at the first one here so the potential failure mode of not enough ink. the cause is all diameters too large a narrow pen angle when writing and not enough

pressure on the pen so those are three of potential mechanisms that we could find there could be more you know there could be less depending again on what item you're looking at so brainstorming these is a very, very important exercise and then again, we're going to look at our ranking in terms of how often does

the root cause of the failure actually occur how often do we think it's going to occur so here's our DFMEA rankings again and we're going to the middle column, and we see some 6s so 6 here and that will take us to once every six months the ball diameter is too big, we think I'll occur once every six months then we have a couple of 4s and that's once every year when we think that would occur and a 3 once every three years

and a 2 here down towards the bottom once every three to six years so again this is what we think as a team how often this mechanism of failure will occur so the next thing, we need to look at is our current design controls and we're thinking about occurrence rather we're thinking about prevention and detection

of our controls we want to make sure that we either have some prevention or detection in place so we have to look at it a little bit differently prevention are things like what is already in place that could possibly prevent the cause and then we look at detection what is already in place that could possibly detect the cause so what do we have to prevent or detect it so what are the test methods or techniques to discover the root cause before the design release so we look at our root cause of failure and what do we have in place here that we could detect or prevent so let's just look at the first one it says big ball diameter is too big we'd have a writing test to detect if problem occurs but then the next one down we have is blank so that's not good we don't have anything in place and so that's going to be a concern and we'll see how that comes up later all right so now we're going to look at the detection how well when you look at our female rankings again so how well do we do are we going to be able to detect this cause so we're in the right-hand column of this DFMEA ranking in the ability detect, 1 is it's obvious and we can definitely find it to 10 we have almost no ability to detect it and so we're looking at our rankings here what we're doing here then as the

first one we gave it a 3 so that would be a statistical process with a hundred percent inspection surrounding out-of-control conditions so that's how we'd find it in some kind of an inspection mode but the next one is 10 is very concerning puts us at the top defect causing failure is not detectable okay that’s not a good sign so let's go on and see what our risk priority number is remember that our risk priority is our severity times

their occurrence times our detection all right here's the rankings again so here's our risk priority number so anything 120 and above we need to take action so if we can see here we have a two eighty and a one and a 126 let's take a and then we have to take some actions let's take a look at that a little bit larger here 280 so that's the highest one remember that was the one where we did it we didn't have any detection or prevention so that's a big concern and we need to find out what kind of recommendations in terms of

action so we need to take action and we need to assign someone and then what we can do with this at first is until we get to actual actions, we can see how our recommend recommended actions would in the interim would possibly impact the severity the currents and detection and thus reduce our RPN that's what we're

essentially after is reducing our issues and reducing our RPN by improving performance we'd also 126 as well because it's 120 and above and we'd be we'd be looking to have some recommend a recommended

actions and someone assigned in a date so here's what the completed DFMEA looks like and 120 and above requires action if you don't have a 120 and above which many students do ask about and my recommendation is that you start with the highest one that you have and look at your top three it's never a bad idea to improve the product to as much as possible so that you don't have any future issues but if you don't have a 120 and above actions for at least  your top three if they're not 120 and above if you have questions about how to fill this out about the actions about any part of the D FMEA please talk to

your instructor or TA.